Anexa 3

Instalarea sistemului  de monitorizare a infrastructurii

Установка системы мониторинга

Содержание

1.	Операционная система.	3

2.	Установка системы мониторинга и соответствующих компонентов.	4

3.	Настройка сервера системы мониторинга.	5

3.1	Настройка apache24.	5

3.2	Настройка Nagios.	6

3.2.1	cfg.conf	6

3.2.2	nagios.cfg	7

3.2.3	resource.cfg	9

3.2.4	commands.cfg	9

3.2.5	contacts.cfg	12

3.2.6	linuxhost.cfg	13

3.2.7	winhost.cfg	16

3.2.8	servicegroup.cfg	18

3.2.9	nagiosgraph.conf	22

3.3	Проверка после настройки Nagios	23

4.	Настройка клиентов.	23

Операционная система.

Так как в организации используются сервера на операционных системах Windows и Unix (FreeBSD). Система мониторинга будет построена на системе Unix (FreeBSD 9.3).

Операционная система FreeBSD установлена на виртуальной среде. Конфигурация 2хXeon процессора, 1024 MB оперативной памяти 40GB HDD пространства, 1 LAN карта (локальный адрес).

Установка происходила в стандартном режиме, через ISO образ. Во время установки был задан пароль root пользователя, создан новый пользователь и настроена сетевая карта (IP, Netmask, Broadcast, DNS)

После установки были выполнены все необходимые обновления. Во FreeBSD 9.1 и более поздние включена программа "самонастройки" ("bootstrap") pkgng. Она скачивает и устанавливает основную утилиту pkgng.

# /usr/sbin/pkg

Действующие инсталляции FreeBSD требуют преобразования базы данных установленных пакетов утилиты pkg_install к новому формату. Для выполнения конвертирования, нужно запустить.

# pkg2ng

Обновление установленных пакетов при помощи pkgng

# pkg upgrade

Обновления безопасности системы.

# freebsd-update fetch

# freebsd-update install

Также был настроен ntp клиент, для синхронизации времени.

В системе были произведены мелкие настройки по визуализации, сменен основной редактор.

Установка системы мониторинга и соответствующих компонентов.

Обновим бинарные исходники.

# pkg update

Так как Nagios использует веб интерфейс и часть интерфейса на php, установим apache c поддержкой php:

# pkg install apache24

# pkg install php5 mod_php5

Для обработки cgi (другая часть интерфейса) установим fcgiwrap:

# pkg install fcgiwrap

Ставим Nagios:

# pkg install nagios

Ставим стандартные плагины для Nagios:

# pkg install nagios-plugins nagios-snmp-plugins nagios-snmp-plugins-extras

Для рисования графиков используем модуль nagiosgraph, так-же нам понадобится rrdtool - набор утилит для работы с RRD:

# pkg install rrdtool nagiosgraph

Настройка сервера системы мониторинга.

Настройка apache24.

Главный конфигурационный файл apache находиться по пути /usr/local/etc/apache24/http.conf.

Приводим его к соответствующему виду:

Для поддержки php, раскоментируем строчку

LoadModule php5_module        libexec/apache24/libphp5.so

Добавим в директиву ifModule

AddType application/x-httpd-php .php

AddType application/x-httpd-php-source .phps

В apache будем использовать виртуальные хосты (http://domainame/virtulhost), для этого раскоментируем строчку

Include etc/apache24/extra/httpd-vhosts.conf

Все остальные параметры остовляем по умолчанию.

Добавим виртуальные хосты для Nagios, /usr/local/etc/apache24/extra/httpd-vhost.conf

Alias /nagios "/usr/local/www/nagios"

<Directory /usr/local/www/nagios/>

Options None

AllowOverride None

AuthName "Nagios Access"

AuthType Basic

AuthUserFile /usr/local/etc/nagios/htpasswd.users

Require valid-user

</Directory>

ScriptAlias /nagios/cgi-bin/ /usr/local/www/nagios/cgi-bin/

<Directory /usr/local/www/nagios/cgi-bin/>

Options ExecCGI

AllowOverride None

AuthName "Nagios Access"

AuthType Basic

AuthUserFile /usr/local/etc/nagios/htpasswd.users

Require valid-user

</Directory>

Создадим файл паролей и заводим пользователя:

# /usr/local/sbin/htpasswd -c /usr/local/etc/nagios/htpasswd.users nagiosadmin

Указываем пароль, подтверждаем пароль.

Для добавления других пользователей параметр –c не используем.

Для поддержки php, переменуем файл php-fpm.conf

cp /usr/local/etc/php-fpm.conf.default php-fpm.conf

Оставим все по умолчанию.

Для запуска apache24 добавим в rc.conf

apache24_enable=”YES”

Настройка Nagios.

В папке /usr/local/etc/nagios/ находятся основные файлы:

cgi.cfg - этот файл в основном содержит разграничение прав доступа к системе через веб.

nagios.cfg - основной конфигурационный файл. Его переименовывать нельзя.

resource.cfg - В стандартном варианте он содержит пути к компонентам.

В папке /usr/local/etc/nagios/objects находятся файлы с описанием всех устройств которых будет обслуживать Nagios. Так как у нас будет больше чем одно устройство, а со временем их может увеличиться до десятков, файлы мы будем создавать, название файла – тип устройства.

commands.cfg - файл, в котором описаны команды вызова модулей и оповещений. При добавлении нового модуля его надо прописывать именно туда.

contacts.cfg – для того, чтобы получать оповещения о состоянии хостов и сервисов.

linuxhost.cfg – файл, в котором будут описаны все устройства на базе Linux-Unix.

localhost.cfg – файл, в котором описан сам сервер с Nagios.

winhost.cfg – файл, в котором будут описаны все устройства на базе Windows.

servicegroup.cfg – файл, для группировки сервисов. Для удобства просмотра определенного сервиса в который входят определенные хосты.

templates.cfg – файл, в котором описываются шаблоны. Шаблоны служат для оформления общей логики для таких сущностей как контакты, сервисы, хосты и т.д.

timeperiods.cfg – файл, в котором описывается временной период для оповещении о проблемах.

upshost.cfg – файл, в котором будут описаны все устройства UPS.

switch.cfg - файл, в котором будут описаны все устройства SWITCH.

hostextinfo.cfg – файл, в котором привязываются иконки для определенных хостов.

В папке /usr/local/etc/nagios /nagiosgraph находятся файлы, в которых описывается все что связано с графическими рисунками. Нас интересует два файла:

nagiosgraph.conf – основной файл.

map – файл, для отображения определенных параметров на рисунках.

Приведем содержимое файлов к следующему виду:

cfg.conf

main_config_file=/usr/local/etc/nagios/nagios.cfg

physical_html_path=/usr/local/www/nagios

url_html_path=/nagios

show_context_help=0

use_pending_states=1

use_authentication=1

use_ssl_authentication=0

authorized_for_system_information=nagiosadmin

authorized_for_configuration_information=nagiosadmin

authorized_for_system_commands=nagiosadmin

authorized_for_all_services=nagiosadmin

authorized_for_all_hosts=nagiosadmin

authorized_for_all_service_commands=nagiosadmin

authorized_for_all_host_commands=nagiosadmin

default_statusmap_layout=5

default_statuswrl_layout=4

ping_syntax=/sbin/ping -n -c 5 $HOSTADDRESS$

refresh_rate=90

result_limit=250

escape_html_tags=1

action_url_target=_blank

notes_url_target=_blank

lock_author_names=1

navbar_search_for_addresses=1

navbar_search_for_aliases=1

nagios.cfg

log_file=/var/spool/nagios/nagios.log

cfg_file=/usr/local/etc/nagios/objects/commands.cfg

cfg_file=/usr/local/etc/nagios/objects/contacts.cfg

cfg_file=/usr/local/etc/nagios/objects/timeperiods.cfg

cfg_file=/usr/local/etc/nagios/objects/templates.cfg

cfg_file=/usr/local/etc/nagios/objects/localhost.cfg

cfg_file=/usr/local/etc/nagios/objects/linuxhost.cfg

cfg_file=/usr/local/etc/nagios/objects/winhost.cfg

cfg_file=/usr/local/etc/nagios/objects/windows_nsclient.cfg

cfg_file=/usr/local/etc/nagios/objects/servicegroup.cfg

object_cache_file=/var/spool/nagios/objects.cache

precached_object_file=/var/spool/nagios/objects.precache

resource_file=/usr/local/etc/nagios/resource.cfg

status_file=/var/spool/nagios/status.dat

status_update_interval=10

nagios_user=nagios

nagios_group=nagios

check_external_commands=1

command_file=/var/spool/nagios/rw/nagios.cmd

lock_file=/var/spool/nagios/nagios.lock

temp_file=/var/spool/nagios/nagios.tmp

temp_path=/tmp

event_broker_options=-1

log_rotation_method=d

log_archive_path=/var/spool/nagios/archives

use_syslog=1

log_notifications=1

log_service_retries=1

log_host_retries=1

log_event_handlers=1

log_initial_states=0

log_external_commands=1

log_passive_checks=1

service_inter_check_delay_method=s

max_service_check_spread=30

service_interleave_factor=s

host_inter_check_delay_method=s

max_host_check_spread=30

max_concurrent_checks=0

check_result_reaper_frequency=10

max_check_result_reaper_time=30

check_result_path=/var/spool/nagios/checkresults

max_check_result_file_age=3600

cached_host_check_horizon=15

cached_service_check_horizon=15

enable_predictive_host_dependency_checks=1

enable_predictive_service_dependency_checks=1

soft_state_dependencies=0

auto_reschedule_checks=0

auto_rescheduling_interval=30

auto_rescheduling_window=180

service_check_timeout=60

host_check_timeout=30

event_handler_timeout=30

notification_timeout=30

ocsp_timeout=5

perfdata_timeout=5

retain_state_information=1

state_retention_file=/var/spool/nagios/retention.dat

retention_update_interval=60

use_retained_program_state=1

use_retained_scheduling_info=1

retained_host_attribute_mask=0

retained_service_attribute_mask=0

retained_process_host_attribute_mask=0

retained_process_service_attribute_mask=0

retained_contact_host_attribute_mask=0

retained_contact_service_attribute_mask=0

interval_length=60

check_for_updates=1

bare_update_check=0

use_aggressive_host_checking=0

execute_service_checks=1

accept_passive_service_checks=1

execute_host_checks=1

accept_passive_host_checks=1

enable_notifications=1

enable_event_handlers=1

process_performance_data=1

service_perfdata_file=/var/spool/nagios/perfdata.log

service_perfdata_file_template=$LASTSERVICECHECK$||$HOSTNAME$||$SERVICEDESC$||$SERVICEOUTPUT$||$SERVICEPERFDATA$

service_perfdata_file_mode=a

service_perfdata_file_processing_interval=30

service_perfdata_file_processing_command=process-service-perfdata

obsess_over_services=0

obsess_over_hosts=0

translate_passive_host_checks=0

passive_host_checks_are_soft=0

check_for_orphaned_services=1

check_for_orphaned_hosts=1

check_service_freshness=1

service_freshness_check_interval=60

service_check_timeout_state=c

check_host_freshness=0

host_freshness_check_interval=60

additional_freshness_latency=15

enable_flap_detection=1

low_service_flap_threshold=5.0

high_service_flap_threshold=20.0

low_host_flap_threshold=5.0

high_host_flap_threshold=20.0

date_format=us

illegal_object_name_chars=`~!$%^&*|'"<>?,()=

illegal_macro_output_chars=`~$&|'"<>

use_regexp_matching=0

use_true_regexp_matching=0

admin_email=admin@urgenta.md

admin_pager=admin@urgenta.md

daemon_dumps_core=0

use_large_installation_tweaks=0

enable_environment_macros=0

debug_level=0

debug_verbosity=1

debug_file=/var/spool/nagios/nagios.debug

max_debug_file_size=1000000

allow_empty_hostgroup_assignment=0

resource.cfg

$USER1$=/usr/local/libexec/nagios

$USER7$=-C community -2

$USER8$=email

$USER9$=password

$USER10$=mail server

commands.cfg

define command{

command_name    notify-host-by-email

command_line    /usr/bin/printf "%b" "***** Nagios *****\n\nNotification Type: $NOTIFICATIONTYPE$\nHost: $HOSTNAME$\nState: $HOSTSTATE$\nAddress: $HOSTADDRESS$\nInfo: $HOSTOUTPUT$\n\nDate/Time: $LONGDATETIME$\n" | /usr/local/bin/sendEmail -s $USER10$ -xu $USER8$ -xp $USER9$ -t $CONTACTEMAIL$ -f $USER8$ -l /var/log/sendEmail -u "** $NOTIFICATIONTYPE$ Host Alert: $HOSTNAME$ is $HOSTSTATE$ **" -m "***** Nagios *****\n\nNotification Type: $NOTIFICATIONTYPE$\nHost: $HOSTNAME$\nState: $HOSTSTATE$\nAddress: $HOSTADDRESS$\nInfo: $HOSTOUTPUT$\n\nDate/Time: $LONGDATETIME$\n"

}

define command{

command_name    notify-service-by-email

command_line    /usr/bin/printf "%b" "***** Nagios *****\n\nNotification Type: $NOTIFICATIONTYPE$\n\nService: $SERVICEDESC$\nHost: $HOSTALIAS$\nAddress: $HOSTADDRESS$\nState: $SERVICESTATE$\n\nDate/Time: $LONGDATETIME$\n\nAdditional Info:\n\n$SERVICEOUTPUT$\n" | /usr/local/bin/sendEmail -s $USER10$ -xu $USER8$ -xp $USER9$ -t $CONTACTEMAIL$ -f $USER8$ -l /var/log/sendEmail -u "** $NOTIFICATIONTYPE$ Service Alert: $HOSTALIAS$/$SERVICEDESC$ is $SERVICESTATE$ **" -m "***** Nagios *****\n\nNotification Type: $NOTIFICATIONTYPE$\n\nService: $SERVICEDESC$\nHost: $HOSTALIAS$\nAddress: $HOSTADDRESS$\nState: $SERVICESTATE$\n\nDate/Time: $LONGDATETIME$\n\nAdditional Info:\n\n$SERVICEOUTPUT$"

}

define command{

command_name    check-host-alive

command_line    $USER1$/check_ping -H $HOSTADDRESS$ -w 3000.0,80% -c 5000.0,100% -p 5

}

define command{

command_name    check_local_disk

command_line    $USER1$/check_disk -w $ARG1$ -c $ARG2$ -p $ARG3$

}

define command{

command_name    check_local_load

command_line    $USER1$/check_load -w $ARG1$ -c $ARG2$

}

define command{

command_name    check_local_procs

command_line    $USER1$/check_procs -w $ARG1$ -c $ARG2$ -s $ARG3$

}

define command{

command_name    check_local_users

command_line    $USER1$/check_users -w $ARG1$ -c $ARG2$

}

define command{

command_name    check_local_swap

command_line    $USER1$/check_swap -w $ARG1$ -c $ARG2$

}

define command{

command_name    check_local_mrtgtraf

command_line    $USER1$/check_mrtgtraf -F $ARG1$ -a $ARG2$ -w $ARG3$ -c $ARG4$ -e $ARG5$

}

define command{

command_name    check_ftp

command_line    $USER1$/check_ftp -H $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_hpjd

command_line    $USER1$/check_hpjd -H $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_snmp

command_line    $USER1$/check_snmp -H $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_http

command_line    $USER1$/check_http -I $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_ssh

command_line    $USER1$/check_ssh $ARG1$ $HOSTADDRESS$

}

define command{

command_name    check_dhcp

command_line    $USER1$/check_dhcp $ARG1$

}

define command{

command_name    check_ping

command_line    $USER1$/check_ping -H $HOSTADDRESS$ -w $ARG1$ -c $ARG2$ -p 5

}

define command{

command_name    check_pop

command_line    $USER1$/check_pop -H $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_imap

command_line    $USER1$/check_imap -H $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_smtp

command_line    $USER1$/check_smtp -H $HOSTADDRESS$ $ARG1$

}

define command{

command_name    check_tcp

command_line    $USER1$/check_tcp -H $HOSTADDRESS$ -p $ARG1$ $ARG2$

}

define command{

command_name    check_udp

command_line    $USER1$/check_udp -H $HOSTADDRESS$ -p $ARG1$ $ARG2$

}

define command{

command_name    check_nt

command_line    $USER1$/check_nt -H $HOSTADDRESS$ -p 12489 -s SED465i -v $ARG1$ $ARG2$

}

define command {

command_name    check_snmp_load_v1

command_line    $USER1$/check_snmp_load.pl -H $HOSTADDRESS$ $USER7$ -T $ARG1$ -w $ARG2$ -c $ARG3$ $ARG4$

}

define command {

command_name    check_snmp_int_v1

command_line    $USER1$/check_snmp_int.pl -H $HOSTADDRESS$ $USER7$ -n $ARG1$ $ARG2$

}

define command{

command_name    check_snmp_process_v1

command_line    $USER1$/check_snmp_process.pl -H $HOSTADDRESS$ $USER7$ -n $ARG1$ -w $ARG2$ -c $ARG3$ $ARG4$

}

define command{

command_name    check_snmp_mem_v1

command_line    $USER1$/check_snmp_mem.pl -H $HOSTADDRESS$ $USER7$ $ARG1$ -w $ARG2$ -c $ARG3$ $ARG4$

}

define command{

command_name    check_disk_snmp

command_line    $USER1$/check_disk_snmp -H $HOSTADDRESS$ -s rourgenta -w 85% -c 90% -d /

}

define command{

command_name    check_snmp_storage_v1

command_line    $USER1$/check_snmp_storage.pl -H $HOSTADDRESS$ $USER7$ -m $ARG1$ -w $ARG2$ -c $ARG3$ $ARG4$

}

define command{

command_name    check_disk_snmp_cache

command_line    $USER1$/check_disk_snmp -H $HOSTADDRESS$ -s rourgenta -w 85% -c 90% -d /cache

}

define command{

command_name    check_disk_snmp_svn

command_line    $USER1$/check_disk_snmp -H $HOSTADDRESS$ -s rourgenta -w 85% -c 90% -d /svn

}

define command{

command_name    process-host-perfdata

command_line    /usr/bin/printf "%b" "$LASTHOSTCHECK$\t$HOSTNAME$\t$HOSTSTATE$\t$HOSTATTEMPT$\t$HOSTSTATETYPE$\t$HOSTEXECUTIONTIME$\t$HOSTOUTPUT$\t$HOSTPERFDATA$\n" >> /var/spool/nagios/host-perfdata.out

}

define command{

command_name    process-service-perfdata

command_line    /usr/bin/perl /usr/local/libexec/nagiosgraph/insert.pl

contacts.cfg

define contact{

contact_name                    Tvetcov         ; Short name of user

use                             generic-contact         ; Inherit default values from generic-contact template (defined above)

alias                           Igori Tvetcov           ; Full name of user

email                           admin@urgenta.md        ; <<***** CHANGE THIS TO YOUR EMAIL ADDRESS ******

}

define contactgroup{

contactgroup_name       admins

alias                   Nagios Administrators

members                 Tvetcov

}

linuxhost.cfg

define host{

use                 remote-server

host_name           proxy

address             xxx.xxx.xxx.xxx

}

define host{

use                 remote-server

host_name           dns

address             xxx.xxx.xxx.xxx

}

define hostgroup{

hostgroup_name    linux-servers

alias             Remote Servers

members           proxy,dns

}

define service{

use                             remote-service         ; Name of service template to use

hostgroups                      linux-servers

service_description             PING

action_url                      /nagios/cgi-bin/nagiosgraph/show.cgi?host=$HOSTNAME$&service=$SERVICEDESC$&geom=450x150

check_command                   check_ping!100.0,20%!500.0,60%

}

define service{

use                             remote-service         ; Name of service template to use

hostgroups                      linux-servers

service_description             SSH

check_command                   check_ssh

notifications_enabled           0

}

define service{

use                             remote-service         ; Name of service template to use

hostgroups                      linux-servers

service_description             CPU Load

check_command check_snmp_load_v1!netsl!4,3,3!8,5,5

}

define service{

use                             remote-service         ; Name of service template to use

hostgroups                      linux-servers

service_description             Check Disk /

check_command                   check_disk_snmp

}

define service{

use                             remote-service         ; Name of service template to use

host_name                       proxy

service_description             Check Disk /cache

check_command                   check_disk_snmp_cache

}

define service{

use                             remote-service         ; Name of service template to use

host_name                       dns

service_description             Check Disk /svn

check_command                   check_disk_snmp_svn

}

define service {

use                             remote-service         ; Name of service template to use

host_name                       proxy

service_description             Lan Ethernet

check_command                   check_snmp_int_v1!"em0"

}

define service {

use                             remote-service         ; Name of service template to use

host_name                       proxy

service_description             Wan Ethernet

check_command                   check_snmp_int_v1!"fxp0"

}

define service {

use                             remote-service         ; Name of service template to use

host_name                       dns

service_description             Lan Ethernet

check_command                   check_snmp_int_v1!"fxp0"

}

define service {

use                             remote-service         ; Name of service template to use

host_name                       dns

service_description             Wan Ethernet

check_command                   check_snmp_int_v1!"fxp1"

}

define service {

use                             remote-service         ; Name of service template to use

host_name                       proxy

service_description             Check Squid

check_command                   check_snmp_process_v1!squid!1,5!0!-2 -m 750,800 -u 90,99

}

define service {

use                             remote-service         ; Name of service template to use

hostgroups                      linux-servers

service_description             Check Memory

check_command                   check_snmp_mem_v1!-N!95,60!99,90

Приведем содержимое файла localhost.cfg к следующему виду:

define host{

use                     freebsd-server          ; Name of host template to use

; This host definition will inherit all variables that are defined

; in (or inherited by) the freebsd-server host template definition.

host_name               localhost

alias                   localhost

address                 xxx.xxx.xxx.xxx

}

define hostgroup{

hostgroup_name  freebsd-servers ; The name of the hostgroup

alias           FreeBSD Servers ; Long name of the group

members         localhost       ; Comma separated list of hosts that belong to this group

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             PING

check_command                   check_ping!100.0,20%!500.0,60%

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             Root Partition

check_command                   check_local_disk!20%!10%!/

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             Current Users

check_command                   check_local_users!20!50

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             Total Processes

check_command                   check_local_procs!250!400!RSZDT

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             Current Load

check_command                   check_local_load!5.0,4.0,3.0!10.0,6.0,4.0

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             Swap Usage

check_command                   check_local_swap!20!10

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             SSH

check_command                   check_ssh

notifications_enabled           0

}

define service{

use                             local-service         ; Name of service template to use

hostgroups                      freebsd-servers

service_description             HTTP

check_command                   check_http

notifications_enabled           0

}

winhost.cfg

define host{

use                windows-server

host_name           dc2.urgenta.md

address             xxx.xxx.xxx.xxx

}

define host{

use                windows-server

host_name           dc.urgenta.md

address             xxx.xxx.xxx.xxx

}

define host{

use                windows-server

host_name           imagistica

address             xxx.xxx.xxx.xxx

}

define host{

use                windows-server

host_name           iis.urgenta.md

address             xxx.xxx.xxx.xxx

}

define host{

use                windows-server

host_name           bd.urgenta.md

address             xxx.xxx.xxx.xxx

}

define host{

use                windows-server

host_name           personal.cnsp.local

address             xxx.xxx.xxx.xxx

}

define host{

use                windows-server

host_name           903.samu.local

address             xxx.xxx.xxx.xxx

}

define hostgroup{

hostgroup_name    windows-servers

alias             Windows Servers

members           dc2.urgenta.md, dc.urgenta.md, iis.urgenta.md, bd.urgenta.md, 903.samu.local, personal.cnsp.local, imagistica

}

define service{

use                     generic-service         ; Name of service template to use

hostgroups              windows-servers

service_description     PING

action_url              /nagios/cgi-bin/nagiosgraph/show.cgi?host=$HOSTNAME$&service=$SERVICEDESC$&geom=450x150

check_command           check_ping!100.0,20%!500.0,60%

}

define service{

use                     generic-service

hostgroups              windows-servers

service_description     Uptime

check_command           check_nt!UPTIME

servicegroups           UPTIME

}

define service{

use                     generic-service

hostgroups              windows-servers

service_description     NTload

check_command           check_nt!CPULOAD!-l 5,80,90

servicegroups           CPU

}

define service{

use                     generic-service

hostgroups              windows-servers

service_description     NTmem

check_command           check_nt!MEMUSE!-w 80 -c 90

servicegroups           MEMORY

}

define service{

use                     generic-service

hostgroups              windows-servers

service_description     NTdiskC

check_command           check_nt!USEDDISKSPACE!-l c -w 80 -c 90

servicegroups           DISK

}

define service{

use                     generic-service

host_name               imagistica

service_description     D:\ Drive Space

check_command           check_nt!USEDDISKSPACE!-l d -w 80 -c 90

servicegroups           DISK

}

servicegroup.cfg

define servicegroup{

servicegroup_name       PING

alias                   Ping check

}

Приведем содержимое файла templates.cfg к следующему виду:

define contact{

name                            generic-contact         ; The name of this contact template

service_notification_period     24x7                    ; service notifications can be sent anytime

host_notification_period        24x7                    ; host notifications can be sent anytime

service_notification_options    w,u,c,r,f,s             ; send notifications for all service states, flapping events, and scheduled downtime events

host_notification_options       d,u,r,f,s               ; send notifications for all host states, flapping events, and scheduled downtime events

service_notification_commands   notify-service-by-email ; send service notifications via email

host_notification_commands      notify-host-by-email    ; send host notifications via email

register                        0                       ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL CONTACT, JUST A TEMPLATE!

}

define host{

name                            generic-host    ; The name of this host template

notifications_enabled           1               ; Host notifications are enabled

event_handler_enabled           1               ; Host event handler is enabled

flap_detection_enabled          1               ; Flap detection is enabled

process_perf_data               1               ; Process performance data

retain_status_information       1               ; Retain status information across program restarts

retain_nonstatus_information    1               ; Retain non-status information across program restarts

notification_period             24x7            ; Send host notifications at any time

register                        0               ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL HOST, JUST A TEMPLATE!

}

define host{

name                            freebsd-server  ; The name of this host template

use                             generic-host    ; This template inherits other values from the generic-host template

check_period                    24x7            ; By default, FreeBSD hosts are checked round the clock

check_interval                  5               ; Actively check the host every 5 minutes

retry_interval                  1               ; Schedule host check retries at 1 minute intervals

max_check_attempts              10              ; Check each FreeBSD host 10 times (max)

check_command                   check-host-alive ; Default command to check FreeBSD hosts

notification_period             workhours       ; FreeBSD admins hate to be woken up, so we only notify during the day

; Note that the notification_period variable is being overridden from

; the value that is inherited from the generic-host template!

notification_interval           120             ; Resend notifications every 2 hours

notification_options            d,u,r           ; Only send notifications for specific host states

contact_groups                  admins          ; Notifications get sent to the admins by default

register                        0               ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL HOST, JUST A TEMPLATE!

}

define host{

name                            remote-server   ; The name of this host template

use                             generic-host    ; This template inherits other values from the generic-host template

check_period                    24x7            ; By default, FreeBSD hosts are checked round the clock

check_interval                  5               ; Actively check the host every 5 minutes

retry_interval                  1               ; Schedule host check retries at 1 minute intervals

max_check_attempts              10              ; Check each FreeBSD host 10 times (max)

check_command                   check-host-alive ; Default command to check FreeBSD hosts

notification_period             workhours       ; FreeBSD admins hate to be woken up, so we only notify during the day

; Note that the notification_period variable is being overridden from

; the value that is inherited from the generic-host template!

notification_interval           120             ; Resend notifications every 2 hours

notification_options            d,u,r           ; Only send notifications for specific host states

contact_groups                  admins          ; Notifications get sent to the admins by default

register                        0               ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL HOST, JUST A TEMPLATE!

}

define host{

name                    windows-server  ; The name of this host template

use                     generic-host    ; Inherit default values from the generic-host template

check_period            24x7            ; By default, Windows servers are monitored round the clock

check_interval          5               ; Actively check the server every 5 minutes

retry_interval          1               ; Schedule host check retries at 1 minute intervals

max_check_attempts      10              ; Check each server 10 times (max)

check_command           check-host-alive        ; Default command to check if servers are "alive"

notification_period     24x7            ; Send notification out at any time - day or night

notification_interval   30              ; Resend notifications every 30 minutes

notification_options    d,r             ; Only send notifications for specific host states

contact_groups          admins          ; Notifications get sent to the admins by default

hostgroups              windows-servers ; Host groups that Windows servers should be a member of

register                0               ; DONT REGISTER THIS - ITS JUST A TEMPLATE

}

define host{

name                    generic-printer ; The name of this host template

use                     generic-host    ; Inherit default values from the generic-host template

check_period            24x7            ; By default, printers are monitored round the clock

check_interval          5               ; Actively check the printer every 5 minutes

retry_interval          1               ; Schedule host check retries at 1 minute intervals

max_check_attempts      10              ; Check each printer 10 times (max)

check_command           check-host-alive        ; Default command to check if printers are "alive"

notification_period     workhours               ; Printers are only used during the workday

notification_interval   30              ; Resend notifications every 30 minutes

notification_options    d,r             ; Only send notifications for specific host states

contact_groups          admins          ; Notifications get sent to the admins by default

register                0               ; DONT REGISTER THIS - ITS JUST A TEMPLATE

}

define host{

name                    generic-switch  ; The name of this host template

use                     generic-host    ; Inherit default values from the generic-host template

check_period            24x7            ; By default, switches are monitored round the clock

check_interval          5               ; Switches are checked every 5 minutes

retry_interval          1               ; Schedule host check retries at 1 minute intervals

max_check_attempts      10              ; Check each switch 10 times (max)

check_command           check-host-alive        ; Default command to check if routers are "alive"

notification_period     24x7            ; Send notifications at any time

notification_interval   30              ; Resend notifications every 30 minutes

notification_options    d,r             ; Only send notifications for specific host states

contact_groups          admins          ; Notifications get sent to the admins by default

register                0               ; DONT REGISTER THIS - ITS JUST A TEMPLATE

}

define service{

name                            generic-service         ; The 'name' of this service template

active_checks_enabled           1                       ; Active service checks are enabled

passive_checks_enabled          1                       ; Passive service checks are enabled/accepted

parallelize_check               1                       ; Active service checks should be parallelized (disabling this can lead to major performance problems)

obsess_over_service             1                       ; We should obsess over this service (if necessary)

check_freshness                 0                       ; Default is to NOT check service 'freshness'

notifications_enabled           1                       ; Service notifications are enabled

event_handler_enabled           1                       ; Service event handler is enabled

flap_detection_enabled          1                       ; Flap detection is enabled

process_perf_data               1                       ; Process performance data

retain_status_information       1                       ; Retain status information across program restarts

retain_nonstatus_information    1                       ; Retain non-status information across program restarts

is_volatile                     0                       ; The service is not volatile

check_period                    24x7                    ; The service can be checked at any time of the day

max_check_attempts              3                       ; Re-check the service up to 3 times in order to determine its final (hard) state

normal_check_interval           10                      ; Check the service every 10 minutes under normal conditions

retry_check_interval            2                       ; Re-check the service every two minutes until a hard state can be determined

contact_groups                  admins                  ; Notifications get sent out to everyone in the 'admins' group

notification_options            w,u,c,r                 ; Send notifications about warning, unknown, critical, and recovery events

notification_interval           60                      ; Re-notify about service problems every hour

notification_period             24x7                    ; Notifications can be sent out at any time

register                        0                      ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL SERVICE, JUST A TEMPLATE!

}

define service{

name                            ups-service             ; The 'name' of this service template

active_checks_enabled           1                       ; Active service checks are enabled

passive_checks_enabled          1                       ; Passive service checks are enabled/accepted

parallelize_check               1                       ; Active service checks should be parallelized (disabling this can lead to major performance problems)

obsess_over_service             1                       ; We should obsess over this service (if necessary)

check_freshness                 0                       ; Default is to NOT check service 'freshness'

notifications_enabled           1                       ; Service notifications are enabled

event_handler_enabled           1                       ; Service event handler is enabled

flap_detection_enabled          1                       ; Flap detection is enabled

process_perf_data               1                       ; Process performance data

retain_status_information       1                       ; Retain status information across program restarts

retain_nonstatus_information    1                       ; Retain non-status information across program restarts

is_volatile                     0                       ; The service is not volatile

check_period                    24x7                    ; The service can be checked at any time of the day

max_check_attempts              3                       ; Re-check the service up to 3 times in order to determine its final (hard) state

normal_check_interval           15                      ; Check the service every 10 minutes under normal conditions

retry_check_interval            1                       ; Re-check the service every two minutes until a hard state can be determined

contact_groups                  admins                  ; Notifications get sent out to everyone in the 'admins' group

notification_options            w,c,r                   ; Send notifications about warning, unknown, critical, and recovery events

notification_interval           120                     ; Re-notify about service problems every hour

notification_period             24x7                    ; Notifications can be sent out at any time

register                        0                      ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL SERVICE, JUST A TEMPLATE!

}

define service{

name                            local-service           ; The name of this service template

use                             generic-service         ; Inherit default values from the generic-service definition

max_check_attempts              4                       ; Re-check the service up to 4 times in order to determine its final (hard) state

normal_check_interval           5                       ; Check the service every 5 minutes under normal conditions

retry_check_interval            1                       ; Re-check the service every minute until a hard state can be determined

register                        0                       ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL SERVICE, JUST A TEMPLATE!

}

define service{

name                            remote-service          ; The name of this service template

use                             generic-service         ; Inherit default values from the generic-service definition

max_check_attempts              4                       ; Re-check the service up to 4 times in order to determine its final (hard) state

normal_check_interval           5                       ; Check the service every 5 minutes under normal conditions

retry_check_interval            1                       ; Re-check the service every minute until a hard state can be determined

register                        0                       ; DONT REGISTER THIS DEFINITION - ITS NOT A REAL SERVICE, JUST A TEMPLATE!

}

nagiosgraph.conf

logfile = /var/spool/nagios/nagiosgraph.log

cgilogfile = /var/log/nagiosgraph-cgi.log

perflog = /var/spool/nagios/perfdata.log

rrddir = /var/spool/nagios/rrd

mapfile = /usr/local/etc/nagios/nagiosgraph/map

nagiosgraphcgiurl = /nagios/cgi-bin/nagiosgraph

nagioscgiurl = /nagios/cgi-bin

javascript = /nagios/stylesheets/nagiosgraph.js

stylesheet = /nagios/stylesheets/nagiosgraph.css

groupdb = /usr/local/etc/nagios/nagiosgraph/groupdb.conf

debug = 5

geometries = 650x50,800x100,1000x200,2000x100

colorscheme = 3

colors = 90d080,30a030,90c0e0,304090,ffc0ff,a050a0,ffc060,c07020

plotas = LINE2

plotasLINE1 = load5,data;load15,data

plotasLINE2 =

plotasLINE3 =

plotasAREA = idle,data;system,data;user,data;nice,data

plotasTICK =

stack = system,data;user,data;nice,data

lineformat = warn=LINE1,D0D050;crit=LINE1,D05050

timeall = day,week,month,year

timehost = day,week,month

timeservice = day,week,month

timegroup = day,week,month

expand_timeall = day,week,month,year

expand_timehost = week

expand_timeservice = week

expand_timegroup = day

timeformat_now = %H:%M:%S %d %b %Y %Z

timeformat_day = %H:%M %e %b

timeformat_week = %e %b

timeformat_month = Week %U

timeformat_quarter = Week %U

timeformat_year = %b %Y

showtitle = true

withmaximums = PING,HTTP

withminimums = PING,HTTP

heartbeat = 600

maximums = Current Load,.*;Current Users,.*;Total Processes,.*;PLW,.*

minimums =

lasts =

dbseparator = subdir

Проверка после настройки Nagios

После всех манипуляций с настройкой Nagios, можно проверить правильность настроек командой /usr/local/bin/nagios -v /usr/local/etc/nagios/nagios.cfg

Для запуска Nagios добавим в файл rc.conf

nagios_enable=”YES”

Запустим все сервисы

service apache24 start

service nagios start

Теперь можно открыть в любом браузере нашу страничку http://dnsnameserver/nagios

Настройка клиентов.

Для Windows клиентов нам понадобится скачать и установить программу NSClient++. При установке указываем IP адрес сервера с установленным Nagios, указываем пароль для windows клиента и ставим галочки на против нужных модулей.

Для Linux и других устройств будем использовать протокол SNMP. Для установки SNMP в FreeBSD воспользуемся коммандой:

# pkg install net-snmp

Для настройки нам нужно указать community – имя, права, адрес.

rocommunity name ip address сервера с Nagios.

SNMP работает по порту 161, так что если используется firewall необходимо открыть соответсвующий порт